##### DETECTION AND CLASSIFICATION OF PERIAPICAL DENTAL X-RAY IMAGES USING IMAGE PROCESSING TECHNIQUE

##### A PROJECT REPORT

###### Submitted by

##### ABINAYA.B [REGISTER NO:211418104006]

GAYATHRI.R [REGISTER NO:211418104061]

INDHUMATHI.M [REGISTER NO:211418104089]

in partial fulfillment for the award of the degree of

BACHELOR OF ENGINEERING

in

COMPUTER SCIENCE AND ENGINEERING

## PANIMALAR ENGINEERING COLLEGE

(An Autonomous Institution, Affiliated to Anna University, Chennai)

MAY 2022

# PANIMALAR ENGINEERING COLLEGE

###### (An Autonomous Institution, Affiliated to Anna University, Chennai)

BONAFIDE CERTIFICATE

Certified that this project report “DETECTION AND CLASSIFICATION OF PERIAPICAL DENTAL X-RAY IMAGES USING IMAGE PROCESSING TECHNIQUE” is the bonafide work of “ABINAYA B (211418104006), GAYATHRI R (211418104061), INDHUMATHI M (211418104089)” who carried out the project work under my supervision.

SIGNATURE				        SIGNATURE

Dr.S.MURUGAVALLI,M.E.,Ph.D.,	        Dr.L.JABASHEELA,MCA.,M.E.,Ph.D.,

HEAD OF THE DEPARTMENT   	        SUPERVISOR

Professor - CSE Department

DEPARTMENT OF CSE, 			           DEPARTMENT OF CSE,

PANIMALAR ENGINEERING COLLEGE,                 PANIMALAR ENGINEERING COLLEGE,

NASARATHPETTAI,			                        NASARATHPETTAI,

POONAMALLEE,				           POONAMALLEE,

CHENNAI-600 123.				           CHENNAI-600 123.

Certified that the above mentioned students were examined in End Semester Project Viva-Voce Examination held on...........................

INTERNAL EXAMINER			               EXTERNAL EXAMINER

DECLARATION BY THE STUDENT

We Abinaya B (211418104006), Gayathri R (211418104061), Indhumathi M (211418104089) hereby declare that this project report titled “DETECTION AND CLASSIFICATION OF PERIAPICAL DENTAL X-RAY IMAGES USING IMAGE PROCESSING TECHNIQUES”, under the guidance of Dr.L.Jabasheela,M.C.A.,M.E.,Ph.D., is the original work done by us and we have not plagiarized or submitted to any other degree in any university by us.

- ABINAYA B

- GAYATHRI R

- INDHUMATHI M

ACKNOWLEDGEMENT

We would like to express our deep gratitude to our respected Secretary and Correspondent Dr.P.CHINNADURAI, M.A., Ph.D. for his kind words and enthusiastic motivation, which inspired us a lot in completing this project.

We express our sincere thanks to our Directors Tmt.C.VIJAYARAJESWARI, Dr.C.SAKTHI  KUMAR,  M.E.,  Ph.D    and      Dr.SARANYASREE   SAKTHI

KUMAR B.E.,M.B.A.,Ph.D., for providing us with the necessary facilities to undertake this project.

We also express our gratitude to our Principal Dr.K.Mani, M.E., Ph.D. who facilitated us in completing the project.

We thank the Head of the CSE Department, Dr. S.MURUGAVALLI, M.E., Ph.D., for the support extended throughout the project.

We would like to thank my guide Dr.L.Jabasheela, MCA., M.E., Ph.D., and all the faculty members of  the Department of CSE for their advice and encouragement for the successful completion of the project.

ABINAYA B

GAYATHRI R

INDHUMATHI M

ABSTRACT

Periapical X-rays show the entire tooth, from the exposed crown to the end of the root and the bones that support the tooth. These X-rays are used to find dental problems below the gum line or in the jaw, such as impacted teeth, tooth fractures, abscesses, tumours and bone changes linked to some diseases. An Image procedure method is a method that is beneficial in involving direct observation of the patient diagnosis of determination. Another aspect of dental imaging is that it can be helpful in the field of biometrics. Human dental image analysis is a challenging and time-consuming process due to the unspecified and uneven structures of various teeth, and hence the manual investigation of dental abnormalities is at par excellence. However, automation in the domain of dental image segmentation and examination is essentially the need of the hour in order to ensure error-free diagnosis and better treatment planning.  The radiographic features of periapical inflammatory lesions vary depending on the time course of the lesion. Because very early lesions may not show any radiographic changes, diagnosis of these lesions relies solely on the clinical symptoms that describes the innovative solution which provides efficient disease detection and deep learning with convolutional neural networks (CNN’s) has achieved great success in the classification of various dental diseases. A variety of neuron-wise and layer-wise visualization methods were applied using a CNN, trained with a publicly available dental disease given image dataset. So, it observed that neural networks can capture the colors and textures of lesions specific to respective diseases upon diagnosis, which resembles human decision-making.

TABLE OF CONTENTS

CHAPTER NO.	                                      TITLE	PAGE NO.

ABSTRACT	      vi

LIST OF TABLES                                                                                        vii

LIST OF FIGURES                                                                                       ix

LIST OF SYMBOLS, ABBREVIATIONS 	       x

1.	INTRODUCTION 	       1

1.1. Overview                                                                                                   2

1.2. Problem Definition                                                                                    3

2.	LITERATURE SURVEY                                                                              4

3.	SYSTEM ANALYSIS	       9

3.1. Existing System	      10

3.2. Proposed system	      11

3.3. Feasibility Study	      11

3.4. Hardware Environment	      13

3.5. Software Environment	      13

4.	SYSTEM DESIGN	      14

4.1. Data Flow Diagram	      15

4.2. UML Diagrams	      17

5.	SYSTEM ARCHITECTURE                                                                       21

5.1. System overview 	      22

5.2. Module Design Specification	      23

5.3. Algorithms 	      26

6.	SYSTEM IMPLEMENTATION	      30

6.1. Coding                                                                                                       31

7.	PERFORMANCE ANALYSIS                                                                     42

7.1. Performance Analysis                                                                               43

7.2. Results and Discussion                                                                             45

8.	CONCLUSION                                                                                              46

8.1. Conclusion and Future Enhancements	      47

APPENDICES	      48

A.1 Sample Screens                                                                                         48

REFERENCES	                                                                                         51

LIST OF FIGURES

FIGURE NO.                TITLE                                                  PAGE.NO

4.1.	                      Dataflow Diagram	15

4.2.	                      Usecase Diagram	17

4.3.	                      Class Diagram	18

4.4.	                      Activity Diagram	19

4.5.	                      Sequence Diagram	20

4.7.	                      System Architecture	22

LIST OF SYMBOLS

| S.NO | NOTATION NAME | NOTATION | DESCRIPTION |
| --- | --- | --- | --- |
| 1. | Class | + public - private # protected Class Name -attribute -attribute +operation +operation +operation | Represents a collection of similar entities grouped together. |
| 2. | Association | Class B Class A nAME Class A Class B | Associations represents static relationships between classes. Roles represents the way the two classes see each other. |
| 3. | Actor | Class A Class A Class B Class B | It aggregates several classes into a single classes. |
| 4. | Aggregation |  | Interaction between the system and external environment |
| 5. | Relation (uses) | uses | Used for additional process communication. |
| 6. | Relation (extends) | EXTENDS | Extends relationship is used when one use case is similar to another use case but does a bit more. |

| 7. | Communication |  | Communication between various use cases. |
| --- | --- | --- | --- |
| 8. | State | State | State of the process. |
| 9. | Initial State |  | Initial state of the object |
| 10. | Final state |  | Final state of the object |
| 11. | Control flow |  | Represents various control flow between the states. |
| 12. | Decision box |  | Represents decision making process from a constraint |
| 13. | Usecase | Usecase | Interact ion between the system and external environment. |

| 14. | Component |  | Represents physical modules which is a collection of components. |
| --- | --- | --- | --- |
| 15. | Node |  | Represents physical modules which are a collection of components. |
| 16. | Data Process/State |  | A circle in DFD represents a state or process which has been triggered due to some event or action. |
| 17. | External entity |  | Represents external entities such as keyboard, sensors, etc. |
| 18. | Transition |  | Represents communication that occurs between processes. |
| 19. | Object Lifeline |  | Represents the vertical dimensions that the object communications. |
| 20. | Message |  | Represents the message exchanged. |

LIST OF ABBREVIATION

DFD – Data Flow Diagram

CNN – Convolutional Neural Network

CHAPTER-1

INTRODUCTION

CHAPTER-1

INTRODUCTION

  - OVERVIEW

Periapical X-rays are used to find dental problems below the gum line or in the jaw, similar as impacted teeth, tooth fractures, abscesses, tumours and bone changes linked to some conditions. The radiographic features of seditious lesions vary depending on the time course of the lesion. Because veritably early lesions may not show any radiographic changes, opinion of these lesions relies solely on the clinical symptoms. It is observed that neural networks can capture the colors and textures of lesions specific to separate conditions upon opinion, which resembles mortal decision-making. to classify dental Disease Classification over static facial images using deep learning ways was developed. This is a complex problem that has formerly been approached several times with different ways. While good results have been achieved using feature engineering, this project concentrated on feature learning, which is one of DL promises. Also feature engineering is not necessary, image pre-processing boosts classification accuracy. Then, it reduces noise on the input data. Currently, Agriculture grounded AI dental disease includes is heavily needed. The result completely based on feature learning does not seem close yet because of a major limitation. Therefore, Disease classification could be achieved by means of deep learning techniques. A variety of neuron-wise and layer-wise visualization methods were applied using a CNN, trained with a publicly available dental disease given image dataset. So, it observed that neural networks can capture the colors and textures of lesions specific to respective diseases upon diagnosis, which resembles human decision-making.

  - PROBLEM DEFINITON

The thing is to develop a deep learning model for dental disease image classification by convolutional neural network algorithm for potentially classifying the results in the form of best accuracy by comparing the CNN infrastructure. Utmost of the affiliated works classifies the condition with three classifiers, i.e. Primary Endo with Secondary Perio, Primary Periodontal Lesion, True Combined Lesions. Whether, the scope in our exploration is to resolve this classes of disfigurement. To classify dental Disease Classification over static facial images using deep learning ways was developed. This is a complex problem that has formerly been approached several times with different ways. While good results have been achieved using feature engineering, this design concentrated on feature learning, which is one of DL pledges. The result completely grounded on feature learning does not feel close yet because of a major limitation. Therefore, Disease classification could be achieved by means of deep learning techniques.

CHAPTER-2

LITERATURE SURVEY

CHAPTER-2

LITERATURE SURVEY

1). Sindu Divakaran, Suja D, “Classification of Digital Dental X-ray Images

UsingMachine Learning” ,2021, IEEE,DOI:10.1109.[15]

Dental conditions like dental anomalies, periapical  and dental caries is adding day by day in children and  adults. Artificial intelligence and neural network with its application in medical imaging is impacting the healthcare industry. Application of the fundamentals of machine learning in  dental imaging is making it easy to segment and classify  images .Based on the efficiency and    performance of  classification algorithms, many of the promising ones like SVM, ANN, KNN are applied on dental data set .

2). Amaar Obaid Hassan, Gregory Y. H. Lip, “Acute Dental Periapical Abscess

and New-Onset Atrial Fibrillation: A Nationwide,   Population-Based   Cohort

Study”,2021, MDPI ,DOI:10.3390.[2]

There are limited data on the relationship of acute dental infections with hospitalization and new-onset atrial fibrillation (AF). This study aims to assess the relationship that is between acute periapical abscess and incident AF. This was a retrospective cohort study from a French national database of cases rehabilitated in 2013 (3.4 million patients) with at least five years of follow up. In total, 3,056,291 grown-ups (55.1% lady) needed hospital admission in French hospitals in 2013 while not having a history of AF. Of 4693 cases classified as having dental periapical abscess, 435 (9.27%) developed AF, contrast to 326,241 (10.69%) without dental periapical abscess that developed AF over a mean follow-up of 4.8 ± 1.7 years.

3). Anuj Kumar, “Descriptive  analysis of  dental X-ray images  using  various

practical methods” ,2021, PeerJ, DOI:10.7717.[3]

In dentistry, interpreters interpret numerous dental X-ray imaging modalities to identify tooth-related problems, abnormalities, or teeth structure changes. Another aspect of dental imaging is that it can be helpful in the plot of biometrics. Human dental image analysis is a challenging and time-consuming process due to the unidentified and uneven structures of various teeth, and hence the manual disquisition of dental abnormalities is at par excellence.

4). Kasra   KARAMIFAR, Afsoon   TONDARI,   Mohammad Ali     SAGHIRI,

“Endodontic Periapical Lesion: An Overview on the   Etiology,  Diagnosis    and

Current    Treatment    Modalities”,   2020,    EUROPEAN    ENDODONTIC

DOI:10.14744.[7]

Nonsurgical and surgical endodontic treatments have a highest success rate in the treatment and prevention of apical periodontitis when carried out according to standard and accepted clinical principles. Nevertheless, endodontic periapical lesions remain in some cases.

5). Vicente  Rueda, Ulises Velazquez, “Using the  Periapical Index to evaluate

the  healing of periapical lesions after  root  canal  treatment”,2020,  Giornale

Italiano di Endodnzia DOI:10.32067.[17]

Root canal treatment serves to help or cure periapical periodontitis. The aim of our study was to evaluate the remission of periapical lesions radiographically in patients who had experienced root canal treatment.

6). Parkavi. B, Roshini. T.J, Maheswari. M, “Segmentation of Periapical Dental

X-Ray Images by Applying Morphological Operation”, 2019, IFERP,Volume 5

Segmentation of Dental X-ray pictures is done by using various image processing ways that are helpful in medical diagnosis, clinical functions and time period operations. These ways aim to outline the segmentation of various tooth structures present within the Dental X-ray pictures which can be used for the first discovery of decay, tooth fractures,  passage treatment.

7). Jiafa Mao   , Kaihui   Wang,  Yahong   Hu,   Weiguo   Sheng,    Qixin Feng,

“Grabcut Algorithm for dental X-RAY    images    based on    full      threshold

Segmentation”, 2018, IET Volume 12.[6]

Teeth are delicate to be destroyed due to their corrosion resistance, high melting point and hardness. Dental biometrics can thus give assistance in human forensic identification, especially to the unknown corpses. One of the key issue in dental based Mortal identification is the segmentation of Dental X-ray images. In this paper, a new segmentation algorithm has been proposed for this purpose.

8). Sibel Koçak, Mustafa Murat, “Periapical Health Related to the Quality of

Coronal  Restorations  and  Root  Fillings in a  Turkish  Population” ,  2013,

CumhuriyetDent J DOI:10.7126.[14]

The aim of the present study was to estimate the quality of root canal treatments and coronal restorations investigating their influence on the periapical status of endodontically-treated teeth in a Turkish population grounded on radiographic.

9).  Abdolvahab Ehsani Rad,  Rosely KumoiMohd  Shafry   Mohd Rahim,  Alireza

Norouzi ,   “   Dental   X-Ray    Image    Segmentation   and    Multiple    Feature

Extraction”,  2012, AWERProcedia, DOI:10.1340.[1]

The thing of this research is to automate the process of representation and extracting textural features of dental x-ray images to use in further operations. In order to gain this aim we need to automate the process of the dental x-ray images segmentation and distinguish the teeth from background and other tissues. The segmentation of dental x-ray images could be delicate due to the shape variation and intensity variation within the same dental x-ray images and from one image to another. There are enormous researches about dental image analysis systems but it has to be estimated to find the appropriate method.

10). P.N.R. Nair, “Pathogenesis   of   apical   periodontities  and   the   causes  of

Endodontic Failures” ,2004, [11].

Apical periodontitis is a effect to endodontic infection and manifests itself as the host defense response to microbial challenge expiring from the root canal system. It is viewed as a dynamic hassle between microbial factors and host defenses at the interface between infected radicular pulp and periodontal ligament that results in original inflammation, resorption of hard tissues, destruction of other periapical tissues, and eventual formation of various histopathological orders of apical periodontitis, generally referred to as periapical lesions. The treatment of apical periodontitis, as a disease of root canal infection, consists of eradicating microbes or mainly reducing the microbial load from the root canal and preventing reinfection by orthograde root stuffing.

### CHAPTER-3

### SYSTEM ANALYSIS

### CHAPTER-3

### SYSTEM ANALYSIS

### 3.1. EXISTING SYSTEM

Dental conditions like dental anomalies, periapical and dental caries is adding day by day in children and grown-ups. Artificial intelligence and neural network with its application in medical imaging is impacting the healthcare industry. X –ray imaging is the generally employed technique to diagnose condition of the teeth. Segmentation of differing dental anomalies using neural network is proving to be a boon to the dental field. Application of neural algorithms aids in carrying images with better discovery accuracy.It describes the experimental results of the classification methods using digital dental X ray images for different classes of dental conditions. Dental images, belonging to various dental conditions like vertical impaction, periapical abscess, distal pulp horn caries, missed canal in root canal, etc are stored in the dental image database Automated discovery reduces the workload of a dentist with classification being accurate. In this work, it is suggested that by exercising GLCM features and SVM, KNN and classifiers, the teeth affected by dental caries can be set apart from the normal teeth in a more detailed manner.

Drawback

- It has not concentrated on identifying other dental conditions with CNN as classifier.

- It has not concentrated on increasing the recognition.

### 3.2. PROPOSED SYSTEM

To classify the dental condition. We planned to design deep learning fashion so that a normal person can also be suitable to use it fluently. It proposed system to analyze dental conditions. It tells about the experimental analysis of our methodology. Different number of images is collected for each conditions that was classified into database images and input images. The primary attributes of the image are relied upon the shape and texture acquainted features. The sample screenshots displays the dental disease discovery using color grounded segmentation model.

Advantages

- Reducing subjectiveness arising from human experts in detecting the dental conditions.

- It is essential to detect a particular disease. In our country, numerous are not so educated to get correct information about all conditions.

3.3. FEASIBILITY STUDY

1. TECHNICAL FEASIBILITY

PYTHON

While complex algorithms and versatile workflows stand behind ML and AI, Python's simplicity allows developers to write reliable systems. Developers get to put all their effort into solving an ML issues instead of focusing on the technical nuances of the language.

2. ECONOMIC FEASIBILITY

Economical Feasibility study using COCOMO model was carried out to find the cost/ benefits analysis of the project .

Advanced COCOMO model with organic, semi-detached categories of software project modelling is used.

| MODULE | CATEGORY | EFFORT (PM) | DEVELOPMENT TIME (Months) |
| --- | --- | --- | --- |
| MANUAL NET | SEMI-DETACHED | 3.0(KLOC)1.12 | 2.5(Effort)0.35 |
| ALEXNET | ORGANIC | 2.4(KLOC)1.05 | 2.5(Effort)0.38 |
| LENET | ORGANIC | 2.4(KLOC)1.05 | 2.5(Effort)0.38 |

Manual Net

Effort Adjustment Factor EAF = 1

Estimation of development effort:

Effort = 3.0(KLOC)1.12 PM = 3 x 0.3583 = 1.07 PM

Estimation of development time:

Tdev = 2.5(Effort)0.35 Months = 2.5 x 1.025 = 2.56 Months

Alexnet

Estimation of development effort:

Effort = 2.4(KLOC)1.05 PM = 2.4(0.15)1.05 = 2.4 x 0.1575 =0.38 PM

Estimation of development time:

Tdev = 2.5(Effort)0.38 Months = 2.5(0.38)0.38 = 2.5 x 0.1444 =0.37 Months

Lenet

Estimation of development effort:

Effort = 2.4(KLOC)1.05 PM = 2.4(0.15)1.05 = 2.4 x 0.1575 =0.38 PM

Estimation of development time:

Tdev = 2.5(Effort)0.38 Months = 2.5(0.38)0.38 = 2.5 x 0.1444 =0.37 Months

Total effort = 1.07+0.38+0.38 = 1.83 PM

Total development time = 2.56+0.37+0.37 = 3.3 Months

3. SOCIAL FEASIBILITY

It helps the people with easy identification of stage of disease. It takes less time and it is inexpensive. Because of early prediction it is highly useful for people. So our project is socially feasible.

3.4. HARDWARE REQUIREMENTS

Processor        : Intel i5

Hard disk 	: Minimum 80 GB

RAM         	: Minimum 2 GB

3.5. SOFTWARE REQUIREMENTS

Operating System    : Windows

Development Tool   : Anaconda with Jupyter Notebook

CHAPTER-4

SYSTEM DESIGN

CHAPTER-4

SYSTEM DESIGN

4.1. DATAFLOW DIAGRAM

A data flow diagram (DFD) is a graphical representation of the "inflow" of data through an information system, modeling its process aspects. A DFD is frequently used as a primary step to create an overview of the system without going into great detail, which can latterly be developed.

dental

Disease Recognition

Image

Disease Classification

FIG 4.1. Level 0 DFD Diagram of Dental Disease Classification

Disease Prediction

Test image

dental disease prediction

Feature extraction

Training dataset

FIG 4.2. Level 1 DFD Diagram of Dental Disease Classification

dental     disease prediction

CNN Model

Testing dataset

Training dataset

FIG 4.3. Level 2 DFD Diagram of Dental Disease Classification

Classified dental disease

Dense CNN

CNN Model

dental Feature

FIG 4.4. Level 3 DFD Diagram of Dental Disease Classification

4.2. USECASE DIAGRAM

Use case diagrams are considered for high position requirement analysis of a system. So when the conditions of a system are analyzed the functionalities are captured in use cases. So, it can say that uses cases are nothing but the system functionalities written in an systematized manner.

FIG 4.5. USECASE Diagram of Dental Disease Classification

4.3. CLASS DIAGRAM

Class diagram is basically a graphical representation of the stationary view of the system and represents different aspects of the operation. So a collection of class illustration represent the whole system. The name of the class illustration should be meaningful to describe the aspect of the system. Each element and their connections should be identified in advance Responsibility (attributes and methods) of each class should be easily identified for each class minimal number of properties should be specified and because, unnecessary properties will make the illustration complicated.

FIG 4.6. CLASS Diagram of Dental Disease Classification

4.4. ACTIVITY DIAGRAM

Activity is a particular operation of the system. Activity diagrams are not only used for imaging dynamic nature of a system but they are also used to construct the executable system by using forward and reverse engineering ways. The only missing thing in activity diagram is communication part. It does not show any communication inflow from one activity to another.

FIG 4.7. ACTIVITY Diagram of Dental Disease Classification

4.5. SEQUENCE DIAGRAM

Sequence diagrams model the inflow of logic within your system in a visual manner, enabling you both to document and validate your logic, and are commonly used for both analysis and design purposes. Sequence diagrams are the most popular UML artifact for dynamic modelling, which focuses on identifying the behaviour within your system.

FIG 4.8. SEQUENCE Diagram of Dental Disease Classification

CHAPTER-5

SYSTEM ARCHITECTURE

CHAPTER-5

SYSTEM ARCHITECTURE

5.1. SYSTEM OVERVIEW

In the System Architecture the input will be given in the form of X-Ray images and by collecting the data from the dataset.

Then, with the help of Feature Extraction and by Future Selection Model on Deep Learning the machine will be trained with these models.

And by the Django Framework we are predicting the results whether the patient is affected by the disease or not.

FIG 5.1. System Architecture of Dental Disease Classification

5.2. MODULE DESCRIPTION

LIST OF MODULES

1.  Import The Given Image From Dataset

2.  To Train by Given Image Dataset

3.  Dental Disease Classifications

1. Import The Given Image From Dataset

## We have to import our data set using keras preprocessing image data creator function also we create size, rescale, range, zoom range, horizontal flip. Also we import our image dataset from folder through the data creator function. Then we set train, test, and confirmation also we set target size, batch size and class-mode from this function we have to train using our own created network by adding layers of CNN.

Primary Endo with Secondary Perio

FIG 5.1. Primary Endo with Secondary Perio

Primary Periodontal Lesion

FIG 5.2. Primary Periodontal Lesion

True Combined Lesions

FIG 5.3. True Combined Lesions

2. To Train By Given Image Dataset

To train our dataset using classifier and fit creator function also we make training way per epoch’s also total number of epochs, validation data and confirmation way using this data we can train our dataset.

FIG 5.4. To Train By Given Image Dataset

3. DENTAL DISEASE CLASSIFICATIONS

We give input as figure using keras preprocessing package. That input Image converted into array value using pillow and image to array function package. We have formerly classified dental in our dataset. It classifies what are Disease. Also we have to predict our Disease using predict function.

Given dataset

CNN

Model

Feature Extractions

dental Disease Classification

Input image

FIG 5.5.Dental Disease Classification

The dental recognition method is grounded on a two-channel architecture that is suitable to recognize. The dental parts are cropped and uprooted and also used as the input into the inception layer of the CNN. The Training phase involves the feature extraction and classification using CNN.

5.3. ALGORITHM

A Convolutional Neural Network (CNN) is a DL algorithm which can take in an input image, assign significance (learnable weights and impulses) to various aspects/objects in the image and be suitable to differentiate one from the other.

The pre-processing needed in a CNN is much lower as compared to other classification algorithms. While in primitive styles filters are hand-engineered, with enough training, CNN have the ability to learn these filters/characteristics. The architecture of a CNN is similar to that of the connectivity pattern of Neurons in the Mortal Brain and was inspired by the association of the Visual Cortex. Individual neurons respond to stimulants only in a confined region of the visual field known as the open Field. Their network consists of four layers with 1,024 input units, 256 units in the first hidden layer, eight units in the second hidden subcaste, and two output units.

1. Input Layer

Input layer in CNN contain image data. Figure data is represented by three dimensional matrixes. It needs to be reshaped it into a single column. Suppose you have image of dimension 28 x 28 =784, it need to convert it into 784 x 1 before feeding into input.

## 2. Convo Layer

Convo layer is occasionally called feature extractor subcaste because features of the image are get uprooted within this layer. First of all, a part of image is connected to Convo layer to perform complication operation as we saw before and calculating the dot product between open field (it is a original region of the input image that has the same size as that of filter) and the filter. Result of the operation is single integer of  output volume. Also the filter over the coming open field of the same input image by a Stride and do the same operation again. It will repeat the same process again and again until it goes through the whole image. The output will be the input for the coming layer.

FIG 5.6. CNN model trained dataset accuracy

## 3. Pooling Layer

## Pooling layer is used to reduce the spatial volume of input image after complication. It is used between two Convo layers. If it applies FC after Convo layer without applying pooling or maximum pooling, then it will be computationally expensive. So, the maximum pooling is only way to reduce the spatial volume of input image. It has applied maximum pooling in single depth slice with Stride of 2. It can observe the 4 x 4 dimension input will going to reduce to 2 x 2 dimensions.

## 4. Fully Connected Layer (FC)

Fully  connected  layer   involves   weights,  impulses, and  neurons.  It  connects

neurons in one subcaste to neurons in another subcaste. It is used to classify images

between different orders by training.

## 5. Softmax / Logistic Layer

Softmax or Logistic layer is the last subcaste of CNN. It resides at the end of FC

subcaste. Logistic is used for double classification and softmax.

FIG 5.7. CNN model trained dataset loss values

## 6. Output Layer

Output layer contains the marker which is in the form of one-hot encoded. Now

you have a well understanding of CNN.

CHAPTER-6

## SYSTEM IMPLEMENTATION

CHAPTER-6

SYSTEM IMPLEMENTATION

6.1. CODING

MODULE – 1

import os

import numpy as np # linear algebra

import matplotlib.pyplot as plt

# Dl framwork - tensorflow, keras a backend

import tensorflow as tf

import tensorflow.keras.backend as K

from tensorflow.keras.models import Model, Sequential

from tensorflow.keras.layers import Input, Dense, Flatten, Dropout, BatchNormalization

from tensorflow.keras.layers import Conv2D, SeparableConv2D, MaxPool2D, LeakyReLU, Activation

from tensorflow.keras.optimizers import Adam

from tensorflow.keras.preprocessing.image import ImageDataGenerator

from tensorflow.keras.callbacks import ModelCheckpoint, ReduceLROnPlateau, EarlyStopping

from IPython.display import display

from os import listdir

from os.path import isfile, join

from PIL import Image

import glob

from tensorflow.keras.preprocessing.image import ImageDataGenerator

from tensorflow.keras.layers import Convolution2D

from tensorflow.keras.layers import MaxPooling2D

from tensorflow.keras.layers import Flatten

from tensorflow.keras.layers import Dense

import warnings

warnings.filterwarnings('ignore')

dir_name_train_Primary_Endo_with_Secondary_Perio = 'dataset/Train/Primary Endo with Secondary Perio'

dir_name_train_Primary_Periodontal_Lesion = 'dataset/Train/Primary Periodontal Lesion'

dir_name_train_True_Combined_Lesions = 'dataset/Train/True Combined Lesions'

def plot_images(item_dir, n=6):

all_item_dir = os.listdir(item_dir)

item_files = [os.path.join(item_dir, file) for file in all_item_dir][:n]

plt.figure(figsize=(80, 40))

for idx, img_path in enumerate(item_files):

plt.subplot(7, n, idx+1)

img = plt.imread(img_path)

plt.imshow(img, cmap='gray')

plt.axis('off')

plt.tight_layout()

def Images_details_Print_data(data, path):

print(" ====== Images in: ", path)

for k, v in data.items():

print("%s:\t%s" % (k, v))

def Images_details(path):

files = [f for f in glob.glob(path + "**/*.*", recursive=True)]

data = {}

data['images_count'] = len(files)

data['min_width'] = 10**100  # No image will be bigger than that

data['max_width'] = 0

data['min_height'] = 10**100  # No image will be bigger than that

data['max_height'] = 0

for f in files:

im = Image.open(f)

width, height = im.size

data['min_width'] = min(width, data['min_width'])

data['max_width'] = max(width, data['max_height'])

data['min_height'] = min(height, data['min_height'])

data['max_height'] = max(height, data['max_height'])

Images_details_Print_data(data, path)

print("")

print("Trainned data for Primary Endo with Secondary Perio:")

print("")

Images_details(dir_name_train_Primary_Endo_with_Secondary_Perio)

print("")

plot_images(dir_name_train_Primary_Endo_with_Secondary_Perio, 10)

print("")

print("Trainned data for Primary Periodontal Lesion:")

print("")

Images_details(dir_name_train_Primary_Periodontal_Lesion)

print("")

plot_images(dir_name_train_Primary_Periodontal_Lesion, 10)

print("")

print("Trainned data for True Combined Lesions:")

print("")

Images_details(dir_name_train_True_Combined_Lesions)

print("")

plot_images(dir_name_train_True_Combined_Lesions, 10)

Classifier=Sequential()

Classifier.add(Convolution2D(32,(3,3),input_shape=(128,128,3),activation='relu'))

Classifier.add(MaxPooling2D(pool_size=(2,2)))

Classifier.add(Flatten())

Classifier.add(Dense(38, activation='relu'))

Classifier.add(Dense(3, activation='softmax'))

Classifier.compile(optimizer='rmsprop',loss='categorical_crossentropy',metrics=['accuracy'])

train_datagen=ImageDataGenerator(rescale=1./255,shear_range=0.2,zoom_range=0.2,horizontal_flip=True)

test_datagen=ImageDataGenerator(rescale=1./255)

training_set=train_datagen.flow_from_directory('dataset/Train',target_size=(128,128),batch_size=32,class_mode='categorical')

test_set=test_datagen.flow_from_directory('dataset/Test',target_size=(128,128),batch_size=32,class_mode='categorical')

epochs = 10

batch_size = 32

#### Fitting the model

history = Classifier.fit_generator(

training_set, steps_per_epoch=training_set.samples // batch_size,

epochs=epochs,

validation_data=test_set,validation_steps=test_set.samples // batch_size)

def graph():

#Plot training & validation accuracy values

plt.plot(history.history['accuracy'])

plt.plot(history.history['val_accuracy'])

plt.title('Model accuracy')

plt.ylabel('Accuracy')

plt.xlabel('Epoch')

plt.legend(['Train', 'Test'], loc='upper left')

plt.show()

# Plot training & validation loss values

plt.plot(history.history['loss'])

plt.plot(history.history['val_loss'])

plt.title('Model loss')

plt.ylabel('Loss')

plt.xlabel('Epoch')

plt.legend(['Train', 'Test'], loc='upper left')

plt.show()

graph()

MODULE - 2

# Dl framwork - tensorflow, keras a backend

import tensorflow as tf

import tensorflow.keras.backend as K

from tensorflow.keras.models import Model

from tensorflow.keras.models import Sequential

from tensorflow.keras.layers import Input

from tensorflow.keras.layers import Dense

from tensorflow.keras.layers import Flatten

from tensorflow.keras.layers import Conv2D

from tensorflow.keras.layers import MaxPooling2D

from tensorflow.keras.layers import Dropout

from tensorflow.keras.layers import LeakyReLU

from tensorflow.keras.layers import Activation

from tensorflow.keras.optimizers import Adam

from tensorflow.keras.preprocessing.image import ImageDataGenerator

from tensorflow.keras.callbacks import ModelCheckpoint

from tensorflow.keras.callbacks import ReduceLROnPlateau

from tensorflow.keras.callbacks import EarlyStopping

import warnings

warnings.filterwarnings('ignore')

model = Sequential()

# 1st Convolutional Layer

model.add(Conv2D(filters=96, input_shape=(224,224,3), kernel_size=(11,11), strides=(4,4), padding='valid'))

model.add(Activation('relu'))

# Max Pooling

model.add(MaxPooling2D(pool_size=(2,2), strides=(2,2), padding='valid'))

# 2nd Convolutional Layer

model.add(Conv2D(filters=256, kernel_size=(11,11), strides=(1,1), padding='valid'))

model.add(Activation('relu'))

# Max Pooling

model.add(MaxPooling2D(pool_size=(2,2), strides=(2,2), padding='valid'))

# 3rd Convolutional Layer

model.add(Conv2D(filters=384, kernel_size=(3,3), strides=(1,1), padding='valid'))

model.add(Activation('relu'))

# 4th Convolutional Layer

model.add(Conv2D(filters=384, kernel_size=(3,3), strides=(1,1), padding='valid'))

model.add(Activation('relu'))

# 5th Convolutional Layer

model.add(Conv2D(filters=256, kernel_size=(3,3), strides=(1,1), padding='valid'))

model.add(Activation('relu'))

# Max Pooling

model.add(MaxPooling2D(pool_size=(2,2), strides=(2,2), padding='valid'))

# Passing it to a Fully Connected layer

model.add(Flatten())

# 1st Fully Connected Layer

model.add(Dense(4096, input_shape=(224*224*3,)))

model.add(Activation('relu'))

# Add Dropout to prevent overfitting

model.add(Dropout(0.4))

# 2nd Fully Connected Layer

model.add(Dense(4096))

model.add(Activation('relu'))

# Add Dropout

model.add(Dropout(0.4))

# 3rd Fully Connected Layer

model.add(Dense(1000))

model.add(Activation('relu'))

# Add Dropout

model.add(Dropout(0.4))

# Output Layer

model.add(Dense(3))

model.add(Activation('softmax'))

model.summary()

# Compile the model

model.compile(loss = 'categorical_crossentropy', optimizer='adam', metrics=['accuracy'])

train_datagen=ImageDataGenerator(rescale=1./255,shear_range=0.2,zoom_range=0.2,horizontal_flip=True)

test_datagen=ImageDataGenerator(rescale=1./255)

training_set=train_datagen.flow_from_directory('dataset/Train',target_size=(224,224),batch_size=32,class_mode='categorical')

test_set=test_datagen.flow_from_directory('dataset/Test',target_size=(224,224),batch_size=32,class_mode='categorical')

epochs = 10

batch_size = 32

#### Fitting the model

history = model.fit(

training_set, steps_per_epoch=training_set.samples // batch_size,

epochs=epochs,

validation_data=test_set,validation_steps=test_set.samples // batch_size)

import matplotlib.pyplot as plt

def graph():

#Plot training & validation accuracy values

plt.plot(history.history['accuracy'])

plt.plot(history.history['val_accuracy'])

plt.title('Model accuracy')

plt.ylabel('Accuracy')

plt.xlabel('Epoch')

plt.legend(['Train', 'Test'], loc='upper left')

plt.show()

# Plot training & validation loss values

plt.plot(history.history['loss'])

plt.plot(history.history['val_loss'])

plt.title('Model loss')

plt.ylabel('Loss')

plt.xlabel('Epoch')

plt.legend(['Train', 'Test'], loc='upper left')

plt.show()

graph()

print("[INFO] Calculating model accuracy")

scores = model.evaluate(test_set)

print(f"Test Accuracy: {scores[1]*100}")

MODULE – 3

from tensorflow.keras.callbacks import ModelCheckpoint, ReduceLROnPlateau, EarlyStopping

from tensorflow.keras.models import Sequential

from tensorflow.keras.layers import Convolution2D

from tensorflow.keras.layers import MaxPooling2D

from tensorflow.keras.layers import Dense

import warnings

warnings.filterwarnings('ignore')

Classifier=Sequential()

Classifier.add(Convolution2D(32,3,3,input_shape=(225,225,3),activation='relu'))

Classifier.add(MaxPooling2D(pool_size=(2,2)))

Classifier.add(Convolution2D(128,3,3,activation='relu'))

Classifier.add(MaxPooling2D(pool_size=(2,2)))

Classifier.add(Flatten())

Classifier.add(Dense(256, activation='relu'))

Classifier.add(Dense(3, activation='softmax'))

Classifier.compile(optimizer='rmsprop',loss='categorical_crossentropy',metrics=['accuracy'])

Classifier.summary()

from tensorflow.keras.preprocessing.image import ImageDataGenerator

train_datagen=ImageDataGenerator(rescale=1./255,shear_range=0.2,zoom_range=0.2,horizontal_flip=True)

test_datagen=ImageDataGenerator(rescale=1./255)

training_set=train_datagen.flow_from_directory('dataset/Train',target_size=(225,225),batch_size=32,class_mode='categorical')

test_set=test_datagen.flow_from_directory('dataset/Test',target_size=(225,225),batch_size=32,class_mode='categorical')

epochs = 20

batch_size = 32

Classifier.fit_generator( training_set, steps_per_epoch=training_set.samples // batch_size,

epochs=epochs,

validation_data=test_set,validation_steps=test_set.samples // batch_size)

import h5py

Classifier.save('dental.h5')

from keras.models import load_model

model=load_model('dental.h5')

import numpy as np

from tensorflow.keras.preprocessing import image

test_image=image.load_img('Primary Periodontal Lesion.jpg',target_size=(225,225))

import matplotlib.pyplot as plt

img = plt.imshow(test_image)

test_image=image.img_to_array(test_image)

test_image=np.expand_dims(test_image,axis=0)

result=model.predict(test_image)

result

prediction = result[0]

classes=training_set.class_indices

classes

prediction=list(prediction)

prediction

classes=['Primary Endo with Secondary Perio','Primary Periodontal Lesion','True Combined Lesions']

output=zip(classes,prediction)

output=dict(output)

output

if output['Primary Endo with Secondary Perio']==1.0 :

print('Primary Endo with Secondary Perio')

elif output['Primary Periodontal Lesion']==1.0:

print('Primary Periodontal Lesion')

elif output['True Combined Lesions']==1.0:

print("True Combined Lesions")

PyCharm:

Views.py

from django.shortcuts import render

from django.http import HttpResponseRedirect

from django.urls import reverse_lazy

from django.views.generic import TemplateView

from employee.forms import EmployeeForm

from django.views.generic import DetailView

from employee.models import Employee

class EmployeeImage(TemplateView):

form = EmployeeForm

template_name = 'emp_image.html'

def post(self, request, *args, **kwargs):

form = EmployeeForm(request.POST, request.FILES)

if form.is_valid():

obj = form.save()

return HttpResponseRedirect(reverse_lazy('emp_image_display', kwags={'pk': obj.id}))

context = self.get_context_data(form=form)

return self.render_to_response(context)

def get(self, request, *args, **kwargs):

return self.post(request, *args, **kwargs)

class EmpImageDisplay(DetailView):

model = Employee

template_name = 'emp_image_display.html'

context_object_name = 'emp'

def cancer(request):

result1 = Employee.objects.latest('id')

import numpy as np

import tensorflow as tf

from tensorflow import keras

import h5py

models = keras.models.load_model('C:/Users/SPIRO/Desktop/own/2.WORKING/Periapical_Xrays/Deploy/employee/dental.h5')

from tensorflow.keras.preprocessing import image

test_image = image.load_img('C:/Users/SPIRO/Desktop/own/2.WORKING/Periapical_Xrays/Deploy/media/' + str(result1),

target_size=(225, 225))

test_image = image.img_to_array(test_image)

test_image = np.expand_dims(test_image, axis=0)

result = models.predict(test_image)

prediction = result[0]

prediction = list(prediction)

classes=['Primary Endo with Secondary Perio','Primary Periodontal Lesion','True Combined Lesions']

output = zip(classes, prediction)

output = dict(output)

if output['Primary Endo with Secondary Perio'] == 1.0:

a = "Primary Endo with Secondary Perio"

elif output['Primary Periodontal Lesion'] == 1.0:

a = "Primary Periodontal Lesion"

elif output['True Combined Lesions'] == 1.0:

a = "True Combined Lesions"

return render(request, "result.html", {"out": a})

CHAPTER-7

PERFORMANCE ANALYSIS

CHAPTER-7

PERFORMANCE ANALYSIS

7.1. PERFORMANCE ANALYSIS

It finds Accuracy for all the modules and finally the third module LENET gave high accuracy so we have done project using LENET module.

FIG 7.1. Accuracy Calculation for Dental Disease Classification

This graph represents the training and testing we are giving for the machine.

fig 7.2. Graph of Testing And Training for Dental Disease Classification

7.2. RESULTS AND DISCUSSION

In this project, a research to classify dental Disease Classification over static facial images using deep learning ways was developed. This is a complex problem that has formerly been approached several times with different ways. While good results have been achieved using feature engineering, this project concentrated on feature learning, which is one of DL promises. Also feature engineering is not necessary, image pre-processing boosts classification accuracy. Then, it reduces noise on the input data. Currently, Agriculture grounded AI dental disease includes is heavily needed. The result completely based on feature learning does not seem close yet because of a major limitation. Therefore, Disease classification could be achieved by means of deep learning techniques.

CHAPTER-8

CONCLUSION

CHAPTER-8

CONCLUSION

8.1. Conclusion and Future Enhancements

Further enhancement on the network’s accuracy and generalization can be achieved through the ensuing practices. The first one is to use the whole dataset during  optimization. Using batch optimization is more suitable for longer datasets. Another technique is to estimate emotions one by one. This can lead to detect which emotions are more delicate to classify. Eventually, using a larger dataset for training seems beneficial. Still, such a dataset might not exist currently. Using several datasets might be a result, but a careful procedure to normalize them is needed. Eventually, using full dataset for training, pre-training on each dental, and using a larger dataset seem to have the possibility to improve the network’s performance. Therefore, they should be addressed in future research on this topic. Therefore, with further optimization of the dental dataset and improvements in the algorithm, a computer-aided detection system  can be expected to become an effective and efficient method of diagnosing the dental diseases and by using the deep learning concept the third module lenet gave the more accuracy compared to other 2 modules. So by using the lenet we train the machine for more accuracy. Hence we demonstrated that the deep learning CNN algorithm was useful for assessing the diagnosis and predictability.

APPENDICES

A.1. Sample ScreenShots

A.1. Uploading Teeth Image

A.2.  Click on Results

A.3. Specify the Type

REFERENCES

[1]. Abdolvahab Ehsani Rad, Rosely KumoiMohd Shafry Mohd Rahim, Alireza Norouzi , “Dental X-Ray Image Segmentation and Multiple Feature Extraction”, 2012, AWERProcedia DOI:10.13140

[2]. Amaar Obaid Hassan, Gregory Y. H. Lip, “Acute Dental Periapical Abscess    and New-Onset Atrial Fibrillation: A Nationwide, Population-Based Cohort Study” ,2021,MDPI, DOI:10.3390

[3]. Anuj Kumar, “Descriptive analysis of dental X-ray images using various practical methods” ,2021, PeerJ, DOI:10.7717

[4]. Anuj Kumar, H. S. Bhadauria, Nitin Kumar, “Fuzzy Clustering with Level Set Segmentation for Detection of Dental Restoration area”,2018,IEEE.

[5]. E. H. Said, D E M Nassar, G. Fahmy, H. Ammar , “Teeth segmentation In digitized dental X-ray films using mathematical morphology”,2006.

[6]. Jiafa Mao, Kaihui Wang, Yahong Hu, Weiguo Sheng, Qixin Feng, “GrabCut algorithm for dental X-ray images based on full threshold segmentation”,2018, IET,Volume 12.

[7]. Kasra KARAMIFAR, Afsoon TONDARI, Mohammad Ali SAGHIRI, “Endodontic Periapical Lesion: An Overview on the Etiology, Diagnosis and Current Treatment Modalities”,2020, EUROPEAN ENDODONTIC DOI:10.14744

[8]. María Prados-Privado, Javier García Villalon, Carlos Hugo Martínez-Martínez and Carlos Ivorra, “Dental Images Recognition Technology and Applications”,2020, DOI:10.2856.

[9]. Parkavi. B, Roshini. T.J, Maheswari. M, “Segmentation of Periapical Dental X-Ray Images by Applying Morphological Operation” 2019, IFERP,Volume 5.

[10]. Park WJ and Jun-Beompark, “History and application of artificial neural network in dentistry” ,2018 dec: 12(4):594-601.

[11]. P.N.R. Nair, “Pathogenesis of apical periodontities and the causes of Endodontic Failures” ,2004.

[12]. Rad A.E, Rahim .M, Shafry .M, Rehman.A, Altameem. A, and Saba. T , “Evaluation of Current Dental Radiographs Segmentation Approaches in Computer aided Applications” ,2013,Volume 30.

[13]. Santhosh B, “Review on emerging techniques on detects oral cancer”, 2015,1(1); 41-6.

[14]. Sibel Koçak, Mustafa Murat, “Periapical Health Related to the Quality of Coronal Restorations and Root Fillings in a Turkish Population”,2013,   CumhuriyetDent J  DOI:10.7126.

[15]. Sindu Divakaran, Suja D, “Classification of Digital Dental X-ray Images Using Machine Learning” ,2021,IEEE, DOI:10.1109.

[16]. Sudhakar. T, Bethanney Janney.J, Haritha. D, Juliet Sahaya. M and Parvathy. V, “Automatic Detection and Classification of Brain Tumor using Image Processing Techniques” ,2017,10(11), 1994-1998.

[17]. Vicente Rueda, Ulises Velazquez, “Using the Periapical Index to evaluate the healing of periapical lesions after root canal treatment”,2020, Giornale Italiano di  Endodnzia, DOI:10.32067.